Technical Specification For Reverse Osmosis

| SUBJECT | FEATURS | REQUREMENTS | POINTS |
| --- | --- | --- | --- |
| System | All should be equipped by | L.P.S | * |
|  |  | H.P.S | * |
|  | functions | Should be on line without any depending to water tank | * |
|  | All system should fane | -Double softener | * |
|  |  | -Food Grade resin | * |
|  |  | -(Automatic or Volumetric automatic)rinsing | * |
|  |  | -Preferably intelligent | * |
|  |  | -Active carbon filter with automatic rinsing | * |
| Material | Hydraulic pump | Should be stainless steel | * |
|  | memberance | Should be synthetic except cellulose acetate | * |
|  | Softher housing and active carbon filter should be mad of | 1)triple / Quadra hull | *1 |
|  |  | 2) Stain less steel | *2 |
|  |  | 3)GRP | *3 |
|  | Reverse osmosis package | -If case of RO system and pre-treatment system are completed as a one case | *4 |
|  |  | -If case of RO system and pre-treatment system are not complete, providing Standard certification for softeners and active carbon filters is mandatory | *5 |
| Certificate & standard | Treatment water should be compatible with | AAMI guide lines | * |
|  | Guarantee | 1 years from instalation | * |
|  | After sale service | 10 years | * |
|  | standard | According to EN | * |
|  |  | 93/42/CEE | * |
| Athers | Number for 30 units Number for 40 units | 600-750 L/H at 15`c | * |
|  |  | TDS > 2000 PPM | * |
|  |  | 1000-1250 L/H at15'c | * |
|  |  | TDS> 2000 PPM | * |

- درصورتيكه جنس بدنه سختي گيرها و فيلتر ذغالي از نوع سه لايه و چند لايه باشد 5 امتياز اضافي به جمع امتيازات اضافه مي گردد

- درصورتيكه جنس بدنه سختي گيرها و فيلتر ذغالي از نوع استنلس استيل باشد 3 امتياز اضافي به جمع امتيازات اضافه مي گردد

- درصورتيكه جنس بدنه سختي گيرها و فيلتر ذغالي از نوع GRP  باشد 2 امتياز اضافي به جمع امتيازات اضافه مي گردد

- اگر دستگاه بصورت يك پكيج واحد طراحي شده است 5 امتياز تعلق مي گيرد

- اگر دستگاه بصورت يك پكيج نيست لازم است براي سختي گيرها و فيلتر كربن اكتيو گواهي استاندارد  ارائه گردد در اين صورت 5 امتيار تعلق مي گيرد.

*وجود مغايرت در هر آيتم باعث حذف كالا خواهد شد.